W94XTU BIOS+TXE Update SOP

Update BIOS + TXE from Win8.1 version to Win7 version (use tools fpt64.efi, AfuEfix64.efi, and MeSetx64.efi)

  - Prepare a blank USB drive.

  - Copy all BIOS files to USB drive.

  - Plug-in AC power.

  - Boot up machine, press F7 key to call boot device menu.

  - Choose “UEFI: Build-in EFI Shell to enter UEFI-Shell environment.

  - Type MeSetx64 then system will reboot automatically, press F7 key to call boot device menu.

  - Choose “UEFI: Build-in EFI Shell to enter UEFI-Shell environment.

  - Type flashtxe

      - System will use fpt64.efi to update TXE block, it is a normal phenomenon to appear below warning message.

Warning: Not all of the file data will be written to flash because

the file is longer than the flash area to be written to!

File: "W54XTUXX.bin"

File Length: 8388608

Write Length: 3145728

      - System will run AfuEfix64 to update BIOS block.

  - System will auto power off when the update procedure is finished.

  - Press power button to power on system (please note the system will auto power off/on for several times to initial TXE), repeat step d and step c to enter UEFI-Shell environment.

  - Type EOL to lock TXE block.

  - Enter CMOS setup to check BIOS version, to load system defaults and save changes then reboot system to finish BIOS+TXE update.